ACCELERATED STABILITY DATA

| Product Name | TOBRAMYCIN OPHTHALMIC OINTMENT USP |  | Storage Condition |  | 40ºC± 2ºC, & 75 % RH± 5% RH |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Batch No | 5A01 |  | Batch Size |  | 28846 x 5 gm (150 kg) |  |  |
| Mfg. Date | 01/2025 |  | Exp. Date |  | 12/2026 |  |  |
| Packing | 5 gm Printed Aluminium Collapsible Tube |  | Charging Date |  | 17/01/2025 |  |  |
| Test | Limit | Initial |  | 1 month |  | 3 month | 6 month |
| Description | White to yellowish white opaque ointment | White opaque ointment. |  | White opaque ointment. |  | White opaque ointment. | White opaque ointment. |
| *Identification 1. Identification A by TLC | Tobramycin appears as a pink spot, and the RF values of the spots of the sample solution and of solution B, respectively, corresponds to those of the standard. | Complies |  | NA |  | NA | Complies |
| *Average net filled weight per tube | 5.0 gm to 5.4 gm | 5.11 gm |  | NA |  | NA | 5.08 gm |
| *Uniformity of filled weight | The average net weight of contents of 10 container is not less than the labelled amount (5.0 gm) and the net weight of the content of any single container is not less than 91.0% (4.55 gm) and not more than 109.0% (5.45 gm) of labelled amount. | 102.20% (5.11 gm) |  | NA |  | NA | 101.60% (5.08 gm) |
| *Metal particles | NMT 50 particle (50µm) in 10 tubes. NMT 8 particle (50µm) in 1 tubes. | 00 particle 00 particle |  | NA |  | NA | 00 particle 00 particle |
| Water | NMT 1.0% w/w | 0.1051% |  | 0.1198% |  | 0.1010% | 0.4886% |
| *Leakage Test | No leakage should be found | No leakage was found |  | NA |  | NA | No leakage was found |

NOTE: * Test to be Performed at Initial & end of Stability Study.

** Test to be performed at 1st & 3rd month interval.

| Test | Limit | Initial | 1 month | 3 month | 6 month |
| --- | --- | --- | --- | --- | --- |
| *Particle Size | Not more than 20 particles have a maximum dimension greater than 25 µm Not more than 02 of these particles have a maximum dimension greater than 50 µm None has a maximum dimension greater than 90µm. | 01 particle 00 particle None particle | NA | NA | 01 particle 00 particle None particle |
| **Microbial contamination test by streak plate method | Should be absent | NA | Absent | Absent | NA |
| *Sterility | Growth or turbidity should not present in the original clear media | Complies | NA | NA | Complies |
| Assay of 1. Tobramycin Sulfate eq. to Tobramycin | 90.0 % to 120.0 % of the label amt | 104.85% | 103.89% | 103.50% | 100.50% |
| A.R. No | -- | FIPAR240745 | AST/006/A | AST/006/B | AST/006/C |
| Date | -- | 17/01/2025 | 09/03/2025 | 19/05/2025 | 18/08/2025 |

NOTE: * Test to be Performed at Initial & end of Stability Study.

** Test to be performed at 1st & 3rd month interval.

CONCLUSION: The product subjected to Accelerated data for 06 months in packing as specified above confirms that the results are meeting the laid down specification & no any significant change observed, and further stability is under process.

LONG TERM STABILITY STUDY DATA

| Product Name | TOBRAMYCIN OPHTHALMIC OINTMENT USP |  |  |  | Storage Condition |  | 30ºC± 2ºC, & 75 % RH± 5% RH |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Batch No | 5A01 |  |  |  | Batch Size |  | 28846 x 5 gm (150 kg) |  |  |
| Mfg. Date | 01/2025 |  |  |  | Exp. Date |  | 12/2026 |  |  |
| Packing | 5 gm Printed Aluminium Collapsible Tube |  |  |  | Charging Date |  | 17/01/2025 |  |  |
| Test | Limit | Initial | 3 month | 6 month |  | 9 month | 12 month | 18 month | 24 month |
| Description | White to yellowish white opaque ointment | White opaque ointment. | White opaque ointment. | White opaque ointment. |  | White opaque ointment. | UNDER PROCESS |  |  |
| *Identification 1. Identification A by TLC | Tobramycin appears as a pink spot, and the RF values of the spots of the sample solution and of solution B, respectively, corresponds to those of the standard. | Complies | NA | NA |  | NA |  |  |  |
| *Average net filled weight per tube | 5.0 gm to 5.4 gm | 5.11 gm | NA | NA |  | NA |  |  |  |
| *Uniformity of filled weight | The average net filled weight of the contents of 10 container is not less than the labelled amount (5.0 gm) and the net weight of the content of any single container is not less than 91.0% (4.55 gm) and not more than 109.0% (5.45 gm) of labelled amount (5.0 gm) | 102.20% (5.11 gm) | NA | NA |  | NA |  |  |  |
| *Metal particles | NMT 50 particle (50µm) in 10 tubes. NMT 8 particle (50µm) in 1 tubes. | 00 particle 00 particle | NA | NA |  | NA |  |  |  |
| Water | NMT 1.0% w/w | 0.1051% | 0.0243% | 0.0154% |  | 0.0641% |  |  |  |
| *Leakage Test | No leakage should be found | No leakage was found | NA | NA |  | NA |  |  |  |

NOTE: * Test to be Performed at Initial & end of Stability Study.

** Test to be performed at 3rd & 6th month of interval.

| Test | Limit | Initial | 3 month | 6 month | 9 month | 12 month | 18 month | 24 month |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| *Particle Size | Not more than 20 particles have a maximum dimension greater than 25 µm Not more than 02 of these particles have a maximum dimension greater than 50 µm None has a maximum dimension greater than 90µm. | 01 particle 00 particle None particle | NA | NA | NA | UNDER PROCESS |  |  |
| **Microbial contamination test by streak plate method | Should be absent | NA | Absent | Absent | NA |  |  |  |
| *Sterility | Growth or turbidity should not present in the original clear media | Complies | NA | NA | NA |  |  |  |
| Assay of 1. Tobramycin Sulfate eq. to Tobramycin | 90.0 % to 120.0 % of the label amt | 104.85% | 104.3% | 107.1% | 104.3% |  |  |  |
| A.R.No | -- | FIPAR240745 | RST/008/A | RST/008/B | RST/008/C |  |  |  |
| Date | -- | 17/01/2025 | 19/05/2025 | 18/08/2025 | 08/11/2025 |  |  |  |

NOTE: * Test to be Performed at Initial & end of Stability Study.

** Test to be performed at 3rd & 6th month of interval.

CONCLUSION: The product subjected to Long Term (Real Time) data 09 months in packing as specified above confirms that the results are meeting the laid down specification & no any significant change observed, and further stability is under process.

Remark:

Total 01 batch charged for Accelerated stability & 01 batch charged for Long term stability condition during the review period. No significant changes observed for charged batches during the review period.

Prepared By QA:    									Reviewed By QA:

(Sign/Date)										(Sign/Date)